Supplementary Material for manuscript:

"A Grid-Independent Methodology for Radiative Flux Mapping and Assessment of Cylindrical Solar Receivers.”

By Sainath A. Waghmare, Gabriel Cardona, and Ramón Pujol-Nadal

Contents of the Repository:

1. PTC_design: Folder containing the PTC designs.

  - 70_1740_23040000_Simple.FCStd: PTC geometry file constructed in FreeCAD.

  - Spectral_materials.zip — Spectral optical material files for all four components in OTSunWebApp format:

    - Reflector (mirror): T_Ag_E1 (Ag-BK7)

    - Absorber: thinfilm_absorber_TiAlN_TiAlON_Si3N4 (TiAlN-TiAlON-Si₃N₄)

    - Glass envelope: T_Glass (BK7)

    - Anti-reflective coating: AR1 (MgF₂)

  - ASTMG173_direct.csv — Spectral direct normal irradiance from ASTM G173-03, 290–3990 nm at 20 nm intervals, used for per-ray energy weighting.

OTSun  https://otsunapp.uib.es/OTSunApp/

Upload 70_1740_23040000_Simple.FCStd, Spectral_materials.zip

Put Incident angle 0, desired Nλ, and CSR (0.05).

Obtain ray data file (Th_points_absorber) per Nλ

2. MCRT_RayHit_Data: Raw ray-hit output files from OTSun spectral MCRT simulations of the LS-3 receiver at solar incidence angle = 0°, Buie circumsolar model CSR = 0.05, DNI = 900.07 W/m².

Each file contains absorbed ray records: position (x, y, z), surface normal, wavelength, and attenuation factor (ε) per ray.

- Contains 1 folder: Delta_20_10000. Nλ  = 10000.

- Folder consists of :

  - source_wavelengths: MCRT input data

  - Th_integral_spectrum: Power to absorber from source, input irradiance and optical efficiency.

  - Th_points_absorber: Ray dataset.

Note: Ray-hit data of only the working ray count (N_λ = 10,000, N_rays = 1,118,896) is provided. All other ray-hit data files used in the study (N_λ = 500 to 100,000) are too large and will be available from the corresponding author on request. These files can also be regenerated using `70_1740_23040000_Simple.FCStd` with OTSun.

3. Flux_Reconstruction: Python scripts implementing the three flux reconstruction methods. All scripts read from 2_MCRT_RayHit_Data and write standardized CSV output files with four columns: CircumferentialAngle_deg, LongitudinalAxis_m, CellArea_m2, HeatFlux_W_m2.

Edit the OUTPUT_DIR, HITS_FILE, and rays_per_wavelength variables at the top of each script before running.

- BB_Flux_Rec.py — Bin-based histogram reconstruction code.

Export output file: BB_<NθxNs>_<Nλ>.csv for each resolution and ray count.

- k-NN_Flux_Rec.py — k-Nearest Neighbor reconstruction code.

Export output file: KNN_<NθxNs>_<Nλ>.csv for each resolution and ray count.

- KDE_Flux_Rec.py — Gaussian KDE reconstruction code.

Export output file: KDE_<NθxNs>_<Nλ>.csv for each resolution and ray count.

- FFTKDE_Flux_Rec.py — Fast Fourier Transformation KDE reconstruction code.

Export output file: FFTKDE_<NθxNs>_<Nλ>.csv for each resolution and ray count.

The output of the Flux_Reconstruction script has:

- Average flux (qavg): To generate Figure 3

- Power to absorber (EMCRT): To generate Figure 3

- Maximum flux (qmax): To generate Figure 4 (a), Figure 6(a), Figure 7(a) and Figure 5 (by varying k value in k-NN_Flux_Rec.py)

- Noise and RI (%): Figure 4 (b), Figure 6(b), Figure 7(b)

- Flux contours plots for cylindrical and unrolled surface: Figure 8 (a), (b), (c).

- Reconstruction time (seconds): Table 5.

Run the individual file for each result; for example, to obtain the above list 1 to 6 for BB at 20X20 resolution with Nλ = 2000, run BB_Flux_Rec.py and provide the data accordingly. Accordingly, run all Python code files and obtain results to plot figures 3 to 8.

Dependencies: numpy, pandas, scipy, KDEpy, matplotlib, pip install KDEpy --break-system-packages

Note: Reconstruction time (seconds) may vary depending on the computer configuration and the number of CPU cores/units.

4. Flux_Rec_Plot_Data: Excel files containing the raw data behind all figures in the manuscript, enabling reproduction of each figure in Excel or matplotlib without re-running the reconstruction scripts.

- Energy_Avg_Flux.xlsx – To generate Figure 3

- BB_Plot_data.xlsx – To generate Figure 4

- k-NN_Plot_data.xlsx – To generate Figure 5 and 6

- KDE_Plot_data.xlsx – To generate Figure 7

5. CFD_Meshes: OpenFOAM polyMesh directories for the six receiver tube outer surface meshes generated in Salome and converted with ideasUnvToFoam.

- 20_20.unv

- 80_80.unv

- 300_300.unv

- 60_400.unv

- 80_600.unv

- 100_1000.unv

- Mesh_Conversion_info: Steps and script to convert OpenFOAM Polymesh and subfolders.

6. FTP_Analysis: Python scripts implementing the Flux Transfer Performance (FTP) assessment.

- Folder: CSV – Contains 3 folders (BB, k-NN, KDE) – Contain output .csv files obtained from 3. Flux_Reconstruction  for 60X400 grid.

- ftp_analysis.py — Main analysis script. Reads all 18 source CSV files (6 MCRT resolutions × 3 methods) and evaluates all 108 combinations (18 × 6 CFD meshes). Computes five FTP metrics per combination:

Outputs: FTP_all_results.csv, FTP_matrix_BB/kNN/KDE.csv, and four PNG figures.

- plot_ftp_matrix.py — Standalone script to regenerate FTP heatmap figures from the CSV matrix files. Fully editable style (font, colours, DPI). (for users who do not wish to rerun the full analysis):

- FTP_all_results.csv   —108 rows, all metric values, Figure 9

- FTP_matrix_BB.csv   —6×6 heatmap, BB method, Figure 10, Table 3 and Table 4.

- FTP_matrix_kNN.csv —6×6 heatmap, k-NN method, Figure 10, Table 3 and Table 4.

- FTP_matrix_KDE.csv  —6×6 heatmap, KDE method, Figure 10, Table 3 and Table 4.

- plot_data_solver_difficulty.csv, Figure 11.

- plot_data_metrics_summary.csv, Table 4, Figure 9.

Python: Run ftp_analysis.py

- Provide: All 18 csv files for BB, k-NN, KDE for 20X20, 80X80, 300X300, 60X400, 80X400 and 100X1000 resolutions folder CSV

- Provide: All 6 mesh files from folder CFD_Meshes

- Manually put total absorbed power from MCRT, Q_MAX_PHYSICAL_REFERENCE (or keep default in code).

- Edit paths in the USER SETTINGS block (Section 1 of the script) to point to your CSV and mesh directories.

7. Supplementary_Material: Supplementary_Information.pdf